Report on the outcomes of a Virtual Mobility

Action number: CA21119

Grantee name: Ioannidis Panagiotis

Virtual Mobility Details Title: Earth Care validation activities at Pyrgos Greece.

Description of the work carried out during the VM Description of the virtual collaboration and activities carried out during the VM, with focus on the work carried out by the grantee. Any deviations from the initial working plan shall also be described in this section. (max. 500 words) Surface solar irradiance and aerosol optical depth (AOD) were recorded at Pyrgos to coincide with five EarthCARE satellite overpasses during the VM period, on 01/04/2026, 26/04/2026, 21/05/2026, 15/06/2026 and 10/07/2026. Continuous global irradiance was captured using a Kipp & Zonen CMP11 pyranometer sampling every 30 seconds, while AOD was retrieved with a handheld Microtops II sunphotometer. Microtops readings were taken every 10 minutes between 13:00 and 17:00 local time on each measurement day, with the sampling rate stepped up to once per minute between 15:45 and 16:00 local time so that AOD retrievals coincided closely with the satellite pass. Before each session the instruments were installed and levelled, and data were checked in real time throughout to catch quality issues or instrument faults as they arose. The pyranometer and Microtops records collected are currently being processed and quality-controlled ahead of their inclusion in the final calibrated dataset and measurement report.

Description of the VM main achievements and planned follow-up activities Description and assessment of whether the VM achieved its planned goals and expected outcomes, including specific contribution to Action objective and deliverables, or publications resulting from the VM. Agreed plans for future follow-up collaborations shall also be described in this section.

(max. 500 words) This VM met its goal of collecting coincident ground-based solar irradiance and AOD observations at Pyrgos during EarthCARE overpasses, feeding into validation of the mission's radiative flux products against their 10 W/m² accuracy target at the top and bottom of the atmosphere. Combining CMP11 pyranometer irradiance readings with Microtops II AOD retrievals from five overpass dates, the resulting dataset and its accompanying measurement report add to the Mediterranean ground-based monitoring network built under HARMONIA, and feed directly into Objective 3 of Working Group 3 by tying aerosol measurement accuracy to satellite validation needs. The work was conducted jointly with PMOD/WRC, whose complementary high-precision radiometric measurements support the integration of ground-based observations required for EarthCARE Cal/Val activities. Planned next steps include completing quality control and calibration of the dataset, and continuing collaboration with PMOD/WRC and the HARMONIA network on future overpass campaigns and joint analysis of the combined observational record.

This report is submitted by the grantee to the Action MC for approval and for claiming payment of the awarded grant. The Grant Awarding Coordinator coordinates the evaluation of this report on behalf of the Action MC and instructs the Grant Holder Manager to verify the grant documents compliance and proceed with the payment.